VIBEGUARD: COLLEGE FABLAB 3D PRINTING & SLICER GUIDE

Fast Execution Card for Hardware Lead (Amith) & FabLab 3D Printer Technician
Target Rig: 10 Hz / 600 RPM Unbalance Testbed | Total Est. Mass: ~10g PLA (Rs. 40–50) | Total Print Time: ~40 min

CRITICAL TECHNICIAN INSTRUCTIONS (DO NOT SKIP):
1. ZERO SUPPORTS (0%): All parts are 100% self-supporting. Do NOT add support material.
2. WALL PERIMETERS = 4 WALLS (>= 1.6 mm): Guarantees solid plastic around shaft bore and bolt holes.
3. PRINT ORIENTATION: Print all parts flat on their bottom faces (Z=0). Never print on edge.

1. QUICK SLICER SETTING MATRIX

| Parameter | FAB-01: Rotor Cam Arm (VibeGuard_Rotor_Arm_D_Shaft.stl) | FAB-02: ADXL345 Mount (Enclosed/OpenTop) (VibeGuard_ADXL345_Rigid_Mount.stl) | FAB-03: Under-Motor Shim (VibeGuard_N20_Motor_Shim_Pad_*.stl) |
| --- | --- | --- | --- |
| Function | Couples to N20 D-shaft; spins at 600 RPM with M3 unbalance mass | Rigid accelerometer stand; enlarged 21.6x10.5 mm window fits 20.32 mm header & 8 DuPonts | Eliminates 1.0–1.2 mm gap under motor cradle for zero vibration damping |
| Print Orientation | Flat on bottom face (Z=0) | Flat on base foot (Z=0) | Flat on build plate |
| Support Material | NONE (0% supports) | NONE (0% supports — bridges cleanly) | NONE (0% supports) |
| Infill Density | 100% Solid (Centrifugal stability) | 80% to 100% (Rigid acoustic coupling) | 100% Solid (High compressive clamp) |
| Perimeter Walls | Minimum 4 walls (>= 1.6 mm) | Minimum 4 walls (>= 1.6 mm) | 3 walls |
| Layer Height | 0.16 mm (Bore accuracy) | 0.16 mm – 0.20 mm | 0.16 mm – 0.20 mm |
| Est. Time & PLA | ~12 to 14 min ¦ 3.5 g PLA | ~20 to 22 min ¦ 5.5 g PLA | ~3 to 4 min ¦ 0.8 g PLA |

2. KEY HARDWARE COMPATIBILITY FEATURES

• FAB-01 Rotor Arm (3.35mm & 3.40mm Failsafe): Standard model features 3.35 mm D-bore (+0.35 mm shrinkage allowance, chord 2.775 mm) for snug push-fit. Failsafe model (VibeGuard_Rotor_Arm_D_Shaft_3p40mm.stl) features 3.40 mm D-bore (chord 2.825 mm) to prevent jamming under heavy shrinkage. Both preserve 12.0 mm pitch and 3.20 mm M3 hole.

• FAB-02 Sensor Stand with Enlarged Window: Upright M3 mounting holes spaced 15.00 mm pitch at Z=7.0 mm matching Robocraze ADXL345 PCB. Enlarged 21.6 x 10.5 mm rectangular window allows 20.32 mm black header base and 8 square DuPont female connectors (20.8 mm span) to plug through without binding. OpenTop U-channel variant also included.

• FAB-03 Shim Options: 3 options provided (1.0 mm flat, 1.2 mm flat, 1.0 mm cradle with side rails). Amith should print 1.0 mm cradle shim + 1.0 mm flat shim.

3. PRE-HANDOVER CHECKLIST (WHAT AMITH MUST CHECK BEFORE LEAVING LAB)

[ ] 1. D-Shaft Push-Fit: Slide FAB-01 onto N20 motor shaft. Firm hand push-fit without radial play.

[ ] 2. ADXL345 Alignment: Place ADXL345 board against FAB-02 upright wall. Dual M3 holes line up (15.0 mm pitch).

[ ] 3. Wire Window Clearance: Confirm rectangular window is wide open (18x8.5 mm) with no sagging filament strings.

[ ] 4. Base Flatness: Base flanges of FAB-02 and FAB-03 sit flat on a table with zero warping.

[ ] 5. Monolithic Integrity: 100% solid piece — zero layer splitting or delamination.