# VibeGuard: College FabLab 3D Printing Specification & Job Requisition

Project: VibeGuard — Edge AI Vibration Monitoring & Predictive Maintenance System

Institution: Jyothi Engineering College (Autonomous), Cheruthuruthy, Kerala

Department: Computer Science & Engineering / Electronics & Communication Engineering

Facility: College FabLab / 3D Printing & Prototyping Center

Lab Rate: ₹4.00 per gram (Standard College FabLab PLA/PETG Tariff)

Primary Contact: Amith Krishna Das (Hardware & Mechanical Lead)

## 1. Executive Summary for FabLab Operator / Technician

Note:

Context: This project investigates rotational unbalance and structural bearing vibration at low frequencies (10 Hz / 600 RPM). We need precision 3D-printed components to couple a miniature DC gear motor to an eccentric test mass and to rigidly mount an ADXL345 3-axis digital accelerometer.

We request the fabrication of Two Essential Mechanical Parts (and one optional safety shroud) on an FDM printer (Ender-3, Prusa MK3/MK4, or Bambu Lab):

| Part ID | Component Name | Function | Material | Infill | Est. Mass | Est. Cost (@ ₹4/g) | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FAB-01 | 3 mm D-Shaft Eccentric Rotor Arm | Slides onto N20 motor shaft; carries M3 unbalance bolt & washers | PLA / PETG | 80%–100% | 3.5 g | ₹14.00 | CRITICAL (Mandatory) |
| FAB-02 | ADXL345 Rigid Sensor Mounting Bracket | Rigidly couples accelerometer to baseplate without vibration damping | PLA | 60%–80% | 5.5 g | ₹22.00 | HIGH (Recommended) |
| FAB-03 | Perforated Rotor Safety Shroud (Optional) | Protective containment cage over spinning 600 RPM rotor arm | PLA | 20% | 18.0 g | ₹72.00 | Optional (Can use ₹0 clear cup) |
| FAB-04 | FAB-01 Spare / Backup Unit | Duplicate rotor arm for calibration bench backup | PLA / PETG | 80%–100% | 3.5 g | ₹14.00 | Recommended |

- Total Essential Print Package (FAB-01 × 2 + FAB-02): ~12.5 grams $\rightarrow$ Total Expected Cost: ₹50.00

- Total If Including Full Printed Shroud (FAB-01 × 2 + FAB-02 + FAB-03): ~30.5 grams $\rightarrow$ Total Expected Cost: ₹122.00

## 2. Detailed Technical Specifications & CAD Dimensions

### Part FAB-01: 3 mm D-Shaft Eccentric Rotor Arm (Cam)

Top View (mm)

+-----------------------------+

|  ( O )              ( O )   |

+-----------------------------+

|<---- 8 mm ---->|   |< 8 mm >|

Shaft Hub           Mass Hub

|<--------- 20.0 mm --------->|

(Center-to-Center)

Radius r = 12 mm

#### Exact Functional Dimensions

- Motor Shaft Interface (D-Bore Hub):

* Nominal Shaft Diameter: 3.00 mm (N20 Micro Metal Gear Motor output shaft).

* D-Profile Geometry: Circular bore of diameter $3.15\text{ mm}$ (accounting for FDM hole shrinkage) with a flat chord face at $2.65\text{ mm}$ across the flat.

* Hub Outer Diameter ($OD$): $8.50\text{ mm}$.

* Hub Height / Depth: $9.00\text{ mm}$ (covers the full 9.5 mm flat of the N20 shaft).

* Set Screw Hole (Lateral): $2.50\text{ mm}$ through-hole perpendicular to the flat face (to optionally tap an M3 grub screw for positive retention).

- Connecting Arm Linkage:

* Length (Shaft center to weight hole center): $12.00\text{ mm}$ (governing dynamic eccentric radius $r$).

* Width: $7.50\text{ mm}$.

* Thickness: $4.00\text{ mm}$.

- Eccentric Mass Mounting Hole:

* Through-hole Diameter: $3.20\text{ mm}$ (clearance for standard M3 bolt).

* Hub Outer Diameter around hole: $8.00\text{ mm}$.

* Counterbore / Nut Recess (Top Face): Hexagonal recess of $5.60\text{ mm}$ across flats, depth $2.50\text{ mm}$ (to capture an M3 hex nut or Nyloc nut firmly without spinning).

#### Recommended Slicer Settings for FAB-01

- Layer Height: 0.16 mm or 0.20 mm.

- Perimeters / Wall Loops: 4 walls (minimum) — crucial so the shaft bore is solid plastic, not infill.

- Top & Bottom Solid Layers: 4 layers.

- Infill: 80% to 100% (Gyroid or Rectilinear). High density is required to withstand 10 Hz rotational centrifugal force without dynamic flexing.

- Material: PLA or PETG (Black, Grey, or Blue).

### Part FAB-02: ADXL345 Rigid Accelerometer Mounting Bracket

Front View (L-Bracket)

| |  <-- 3.0 mm thick rigid upright

| |  <-- 2x M3 holes spaced 15.0 mm apart (for ADXL345 board)

| |

___| |_______

|_____________|  <-- Stiff base flange (4.0 mm thick) with 2x wood screw holes

#### Exact Functional Dimensions

- Sensor Flange (Vertical Face):

* Height: $22.00\text{ mm}$, Width: $24.00\text{ mm}$, Thickness: $3.50\text{ mm}$.

* Sensor Mounting Holes: Two $3.20\text{ mm}$ diameter holes, spaced exactly $15.00\text{ mm}$ center-to-center horizontally, elevated $10.00\text{ mm}$ from the base.

- Baseplate Mounting Flange (Horizontal Face):

* Depth: $18.00\text{ mm}$, Width: $24.00\text{ mm}$, Thickness: $4.00\text{ mm}$.

* Base Screw Holes: Two $3.50\text{ mm}$ countersunk holes spaced $16.00\text{ mm}$ apart for securing into the wooden baseplate with wood screws.

- Stiffening Gussets: Two triangular 45° gussets ($3.0\text{ mm}$ thick) joining vertical and horizontal walls to prevent high-frequency cantilever resonance.

#### Recommended Slicer Settings for FAB-02

- Layer Height: 0.20 mm.

- Infill: 60% Gyroid.

- Perimeters: 3 to 4 walls.

- Orientation: Print resting on the flat base flange (no support required).

### Part FAB-03: Perforated Safety Containment Shroud (Optional)

Isometric Shroud Wireframe

+-------------------+

/                   /|

+-------------------+ |

|   [ VENT SLOTS ]  | | Height: 35 mm

|   [    ///     ]  | +

|                   |/

+-------------------+

Base: 42 x 38 mm

#### Exact Functional Dimensions

- External Footprint: $42.00\text{ mm} \times 38.00\text{ mm} \times 35.00\text{ mm}$ height.

- Internal Clearance: Guarantees $\ge 6.00\text{ mm}$ clearance around the $12\text{ mm}$ rotating arm + M3 bolt head at all 360° positions.

- Wall Thickness: $1.60\text{ mm}$ (4 perimeters with 0.4 mm nozzle).

- Observation & Airflow Slots: Three $3\text{ mm} \times 20\text{ mm}$ vertical viewing slots on top and side to observe rotation without opening the guard.

- Mounting Lugs: Two $10\text{ mm}$ exterior ears at base with $3.5\text{ mm}$ holes to screw directly onto the wood baseplate.

## 3. Step-by-Step Instructions for FabLab Technician

- CAD File Generation / Import:

* Import the STL/STEP files into PrusaSlicer / Bambu Studio / Cura.

* If generating CAD from scratch in Fusion 360 / SolidWorks / Onshape, use the parametric sketches defined in Section 2.

- Material Selection:

* Preferred: Standard PLA (1.75 mm) — high Young's modulus ($E \approx 3.5\text{ GPa}$) ensures exceptional rigidity for vibration transfer.

* Alternative: PETG — good layer adhesion and impact resistance.

- Print Execution:

* Clean print bed with Isopropyl Alcohol (IPA) to ensure flat first layer without warping.

* Total print duration:

* FAB-01 (Rotor Arm): ~12 minutes per piece.

* FAB-02 (Sensor Bracket): ~18 minutes.

* Total runtime: ~40 minutes for the essential set.

- Post-Processing & Quality Check:

* Bore Fit Check: Take a 3.0 mm steel rod or N20 motor shaft and test the D-profile fit. The rotor arm should slide on snugly by hand with zero radial wobble.

* Deburring: Clean any brim or elephants-foot brim from the bottom of the D-bore using a 3 mm hobby drill bit or craft blade.

## 4. Billing & Requisition Approval Slip

========================================================================

JYOTHI ENGINEERING COLLEGE - FABLAB REQUISITION

========================================================================

Project Title    : VibeGuard (Edge AI Vibration Monitoring Rig)

Academic Course  : PBCST504 / Final Year Embedded Project

Student Name     : Amith Krishna Das & Team

Department       : CSE / ECE

Items Ordered:

1. 3mm D-Shaft Eccentric Rotor Arm (FAB-01 x 2)   :  ~7.0 grams

2. ADXL345 Rigid Sensor Bracket (FAB-02 x 1)      :  ~5.5 grams

3. Optional Safety Shroud (FAB-03 x 1)           : [ ] Check if requested (~18g)

Calculated Weight (Essential Set)                 : 12.5 grams

FabLab Tariff Rate                                : ₹4.00 per gram

------------------------------------------------------------------------

ESTIMATED LAB FEE PAYABLE                         : ₹50.00 (or ₹122.00 w/ shroud)

------------------------------------------------------------------------

Technician Signature : ___________________   Date: ___________________

Lab Supervisor Stamp : ___________________

========================================================================