# VibeGuard: Mechanical Workshop & Carpentry Job Requisition

Component: Rig Baseplate (G4-1) & Mechanical Mounting Preparation

Institution: Jyothi Engineering College (Autonomous), Cheruthuruthy, Kerala

Department: Computer Science & Engineering / Electronics & Communication Engineering

Target Facility: Mechanical Workshop (Carpentry & Fitting Section)

Primary Contact: Amith Krishna Das (Hardware & Mechanical Lead)

## 1. Work Order Overview for Workshop Superintendent / Technician

Note:

Engineering Purpose: VibeGuard is an embedded edge vibration monitor. We are building a benchtop machine fault testbed using a 12V N20 gear motor that spins an off-center unbalance weight.

Physical Requirement: The motor and sensor must be anchored to a stiff, dense, flat plate so mechanical vibration transfers efficiently through the solid material into the ADXL345 sensor without flexing, warping, or acoustic resonance.

## 2. Material Options (Scrap Bin Offcut is Acceptable)

Please provide and cut one piece from workshop scrap stock matching either of these specifications:

| Preference | Material Type | Dimensions ($L \times W \times T$) | Notes |
| --- | --- | --- | --- |
| Option 1 (Best) | Hard Plywood or Marine Plywood | $150 \times 150\text{ mm}$ ($\approx 6 \times 6\text{ inches}$), $10\text{ mm}$ to $12\text{ mm}$ thick | High stiffness, dampens tabletop rattle, holds wood screws firmly. |
| Option 2 | Dense Hardwood Plank (Teak / Mahogany / Rubberwood) | $150 \times 150\text{ mm}$, $12\text{ mm}$ to $15\text{ mm}$ thick | Must be planed flat on both faces. Zero warping. |
| Option 3 | Clear Cast Acrylic Sheet (PMMA) | $150 \times 150\text{ mm}$, $8\text{ mm}$ to $10\text{ mm}$ thick | Excellent aesthetics for exhibition, rigid transmission. |

Caution:

Prohibited Materials: Do NOT use soft cardboard, styrofoam, low-density packing wood, or thin sheet metal ($\le 3\text{ mm}$). Thin metal flexes like a drumhead (membrane resonance), creating spurious harmonics.

## 3. Baseplate Layout & Drilling / Marking Template

+-----------------------------------------------------------------------+

|  [O] 4 mm Mounting Hole (Corner 1)              Corner 2 [O]          |

|                                                                       |

|                     +---------------------------+                     |

|                     |   BREADBOARD & ESP32      |                     |

|                     |   ELECTRONICS ZONE        |                     |

|                     |   (85 mm x 55 mm)         |                     |

|                     +---------------------------+                     |

|                                                                       |

|        N20 MOTOR POSITION                  ADXL345 SENSOR             |

|       +-------------------+               +---------------+           |

|       |  [O]  (17 mm) [O] |               |  [O] (15) [O] |           |

|       |   Motor U-Bracket | <--- 20 mm -->|  Sensor Mount |           |

|       +-------------------+               +---------------+           |

|                 |                                                     |

|                 v                                                     |

|          Rotating Cam Area                                            |

|                                                                       |

|  [O] Corner 3                                   Corner 4 [O]          |

+-----------------------------------------------------------------------+

|<------------------------------ 150 mm ------------------------------->|

### Drilling & Machining Operations Requested:

- Cutting & Squaring:

* Cut baseplate stock to square dimensions: $150\text{ mm} \times 150\text{ mm} \ (\pm 2\text{ mm})$.

* Lightly sand/deburr all 4 edges and corners to prevent splinters.

- Motor Bracket Pilot Holes (2 Holes):

* Location: Center-left quadrant of the board.

* Center-to-Center Spacing: $17.0\text{ mm}$ (matches the pre-punched holes in the aluminum N20 U-bracket).

* Pilot Drill Bit: $2.0\text{ mm}$ or $2.5\text{ mm}$ drill bit to a depth of $8\text{ mm}$ (prevents wood from splitting when tightening M3 wood screws).

- Sensor Mount Pilot Holes (2 Holes):

* Location: Exactly $20.0\text{ mm}$ parallel distance from the motor bracket.

* Center-to-Center Spacing: $15.0\text{ mm}$ (or $16.0\text{ mm}$ for 3D bracket base).

* Pilot Drill Bit: $2.0\text{ mm}$ drill bit to a depth of $8\text{ mm}$.

- Base Isolation Feet (Optional, 4 Holes):

* Four $3.5\text{ mm}$ holes, 10 mm inset from each corner, to attach small rubber feet or standoffs so the rig does not vibrate across the table during tests.

## 4. Requisition Dialogue Script for Students

Whom to Approach: Carpentry Workshop Instructor or Fitting/Machine Shop Superintendent.

"Good morning Sir,

We are final-year students working on our microcontroller semester project (VibeGuard Vibration Fault Diagnostic Rig).

We need a small, sturdy baseboard from the workshop scrap bin to mount our test motor and vibration sensor:

1. Could you help us cut a scrap offcut piece of plywood or dense wood, roughly 15 cm by 15 cm, 10 to 12 mm thick?

2. Could you help us drill two small 2 mm pilot holes with 17 mm spacing so we can screw down our miniature motor bracket?

3. Do you also have 2 or 4 small self-tapping wood screws (around 10 mm length) to fasten the bracket into the wood?

Here is the dimension drawing. Thank you very much, Sir!"

## 5. Acceptance Criteria Checklist (Before Leaving Workshop)

☐ Flatness: Baseplate rests flat on a granite surface plate or workshop table with zero rock/wobble.

☐ Dimensions: Length and width are between $140\text{ mm}$ and $160\text{ mm}$.

☐ Hole Spacing: Bracket mounting holes align cleanly with the aluminum N20 U-bracket slots.

☐ Screws Included: 2 to 4 small wood screws received and tested for grip into the pilot holes.