# VibeGuard: Local Sourcing Action Guide & Rig Assembly Manual

Scope: Group 3 (Passive Electronic Components) & Group 4 (Mechanical Test Rig Assembly)

Target Audience: Amith Krishna Das (Hardware & Rig Lead) & VibeGuard Integration Team

Institution: Jyothi Engineering College (Autonomous), Cheruthuruthy, Kerala

## 1. Complete Bill of Materials (Group 3 & Group 4)

| Item ID | Component Name | Technical Specification / Code | Qty | Target Sourcing Location | Est. Cost |
| --- | --- | --- | --- | --- | --- |
| G3-1 | Electrolytic Bulk Capacitor | 100 µF, 16V or 25V (Radial leads, low ESR) | 2 pcs | College ECE/Hardware Lab OR Local TV Repair Shop | ₹0 – ₹5 |
| G3-2 | Ceramic Bypass Capacitor | 100 nF (0.1 µF), marked "104" (Disc or MLCC) | 2 pcs | College ECE/Hardware Lab OR Local Electronics Shop | ₹0 – ₹2 |
| G3-3 | Flyback Clamp Diode | 1N4007 (1A, 1000V) or 1N5819 (1A, 40V Schottky) | 1 pc | College ECE/Hardware Lab OR Local Electronics Shop | ₹0 – ₹2 |
| G3-4 | Current-Limiting Resistors | 220 Ω, ¼ Watt (Color code: Red-Red-Brown-Gold) | 2 pcs | College ECE/Hardware Lab drawer | ₹0 – ₹2 |
| G4-1 | Rig Baseplate | Flat wood, plywood, MDF, or 8–10 mm acrylic ($\approx 15 \times 15\text{ cm}$) | 1 pc | College Carpentry / Mech Workshop scrap bin | ₹0 |
| G4-2 | Eccentric Weight Bolt | M3 × 15 mm (or M3 × 20 mm) machine screw / hex socket bolt | 1 pc | College Mech Lab OR Local Nut-and-Bolt Shop | ₹1 – ₹2 |
| G4-3 | Metal Washers | M3 Flat Steel Washers (Inner dia: 3.2 mm, Outer dia: 7–9 mm) | 4 pcs | College Mech Lab OR Local Nut-and-Bolt Shop | ₹2 |
| G4-4 | Nylon Locknut | M3 Nyloc Nut (Nut with internal nylon ring to prevent loosening) | 1 pc | College Mech Lab OR Local Nut-and-Bolt Shop | ₹1 – ₹2 |
| G4-5 | Motor Wood Screws | M2.5 or M3 wood screws (length 10–12 mm) to mount U-bracket | 2 pcs | College Mech Lab OR Local Hardware Shop | ₹2 |
| G4-6 | 3 mm Shaft Coupling / Rotor Arm | 3 mm bore hub with set-screw OR 3D-Printed Eccentric Rotor Arm | 1 pc | College FabLab / 3D Printer (Recommended) OR Hobby Store | ₹0 – ₹10 |
| G4-7 | Transparent Safety Guard | Clear plastic enclosure, acrylic box, or transparent cup ($\approx 8 \times 8\text{ cm}$) | 1 pc | Local household shop / Lab scrap plastic container | ₹0 – ₹10 |

## 2. Visual Reference: The Assembled Mechanical Rig

!VibeGuard Mechanical Test Rig Baseplate

Key features shown: (1) Rigid wooden baseplate, (2) Metal N20 mounting bracket bolted to wood, (3) 3 mm shaft hub holding an off-center M3 bolt and washers, (4) ADXL345 sensor bolted rigidly beside the motor, sharing structural vibration.

## 3. Local Sourcing Action Playbook: Where, Whom & What to Ask

### Location A: College Electronics / Hardware Labs (ECE & CSE Departments)

- Target Components: 100 µF capacitor, 100 nF ("104") ceramic capacitor, 1N4007 diode, 220 Ω resistors.

- Whom to Ask: The Lab Assistant, Technical Officer, or Lab in-charge of the Basic Electronics Lab or Microcontroller Lab.

- What to Say (Dialogue Script):

> "Good morning Sir/Madam, we are working on our PBCST504 Microcontroller semester project (VibeGuard vibration monitor). We need a few standard basic passives from the component store if available:

> 1. One or two 100 µF (16V or 25V) electrolytic capacitors for power filtering.

> 2. Two 100 nF ceramic capacitors (the small disc ones marked '104').

> 3. One 1N4007 diode for motor flyback protection.

> 4. Two 220 Ω resistors for our RGB LED.

> Could you please issue these from the lab inventory drawer?"

- If unavailable at college: Any local TV/electronics repair shop or consumer electronics service center in Cheruthuruthy or Shoranur will hand you all four parts from their workbench for ₹10 cash.

### Location B: College Mechanical Workshop & Carpentry Lab

- Target Components: The Wooden Baseplate ($\approx 15 \times 15\text{ cm}$), M3 wood mounting screws, and scrap acrylic.

- Whom to Ask: The Carpentry Workshop Instructor or Mechanical Workshop Superintendent.

- What to Say (Dialogue Script):

> "Sir, we are engineering an embedded machine monitoring rig for our project. We need a small, sturdy flat baseboard to mount a miniature motor and sensor.

> Do you have a small offcut scrap piece of plywood, hardwood, or 8–10 mm thick acrylic sheet, roughly $15 \times 15\text{ cm}$ (or 6 inches square) from the scrap bin? Also, if possible, 2 small wood screws to fasten a small metal bracket?"

- Key Spec: The wood must be flat, dense, and at least 8 mm to 12 mm thick so that vibration from the motor travels through the wood into the sensor without flexing or warping.

### Location C: College FabLab / Makerspace (3D Printing Option)

- Target Components: 3 mm Shaft Eccentric Rotor Arm & ADXL345 Sensor Standoff Bracket.

- Whom to Approach: FabLab Student Coordinator, Makerspace Technician, or 3D Printing Operator.

- What to Request & Exact CAD Specifications:

Ask to print a small Eccentric Vibration Cam/Arm in standard PLA (20% to 30% infill, takes 8–12 minutes):

1. Motor Shaft Bore: A cylindrical hole of $3.0\text{ mm}$ diameter with a flat 'D' profile (matching the N20 D-shaft flat side) so it slides tightly onto the shaft without slipping.

2. Arm Length / Offset Radius ($r$): Distance between the center of the shaft hole and the mass hole = $10\text{ mm}$ to $12\text{ mm}$.

3. Off-Axis Bolt Hole: A $3.2\text{ mm}$ through-hole to pass the M3 bolt.

4. Set Screw Hole (Optional but good): A small 2.5 mm lateral hole on the side to tap an M3 grub screw if a friction fit isn't snug.

- What to Say (Dialogue Script):

> "Hi, I have a tiny 5-gram 3D model for our microcontroller vibration rig. It's a small 3mm D-shaft eccentric cam arm to hold a small M3 bolt for vibration testing. Can we slice and print this on the Ender-3 / Prusa using PLA?"

### Location D: Local Nut-and-Bolt / Hardware Store (Cheruthuruthy / Shoranur / Thrissur)

- Target Components: M3 Machine Bolts, Washers, and Nyloc Locknuts.

- Where to Go: Any local retail hardware store, nut-bolt specialist, or motorcycle/bicycle spare parts shop.

- What to Ask the Shopkeeper:

> "Bhai / Chetta, do you have small metric machine screws? I need:

> • One M3 size, 15 mm length bolt (Allen key or Star head).

> • Four M3 flat steel washers.

> • One M3 Nyloc nut (the nut with the white nylon plastic lock ring inside).

> • Two small self-tapping wood screws (around 10 mm length) to screw a bracket into wood."

- Cost: Typically ₹5 to ₹10 for the entire handful.

## 4. Rig Assembly & Wiring Instructions: What to Do with Each Part

+-------------------------------------------------------+

|                 VIBEGUARD PHYSICAL RIG                |

|                                                       |

|   [ 12V Wall Adapter ]                                |

|            |                                          |

|            v                                          |

|   [ DC Barrel Jack ] ---> [ KCD1 Switch ] ---> [ Fuse ]

|                                                  |    |

|   +----------------------------------------------+    |

|   |                                                   |

|   |        [ 1N4007 Diode (Silver Band to +12V) ]     |

|   |                     |                             |

|   v                     v                             |

|  (+) ---------------------------------- (-)           |

|         N20 12V GEAR MOTOR (600 RPM)                  |

|                      |                                |

|          [ 3mm D-Shaft Hub / Arm ]                    |

|                      |                                |

|          [ M3 Bolt + Washers + Nyloc ]                |

|          (Centrifugal Vibration: 10 Hz)               |

|                      |                                |

|      [ RIGID WOODEN BASEPLATE (15x15 cm) ]            |

|                      |                                |

|                      v                                |

|             [ ADXL345 SENSOR ]                        |

|                      | (4-Wire SPI)                   |

|                      v                                |

|             [ 7Semi ESP32 MCU ] <--- 5V USB (PC)      |

|                      |                                |

|         [ 100uF + 100nF Caps across 3V3/GND ]         |

+-------------------------------------------------------+

### Step 1: Baseplate & Motor Mounting (G4-1 & G4-5)

- Place the flat wooden baseplate on a level desk.

- Position the metal N20 U-bracket roughly in the center-left of the board.

- Slide the N20 motor inside the bracket and tighten the bracket's clamping screw around the brass gearbox.

- Use the two small wood screws (G4-5) to fasten the bracket firmly into the wooden plank. Ensure the motor does not wiggle or tilt.

### Step 2: Assembling the Eccentric Mass Rotor (G4-2, G4-3, G4-4, G4-6)

- Take the 3D-printed rotor arm (or 3 mm metal hub).

- Insert the M3 × 15 mm bolt through the off-axis hole.

- Thread 3 to 4 steel washers onto the bolt. These washers provide the off-center unbalance mass ($m \approx 2.5\text{ g}$).

- Screw on the M3 Nyloc nut and tighten firmly with a spanner/pliers.

> [!IMPORTANT]

> A standard plain nut can vibrate loose within 30 seconds of motor rotation. The nylon ring inside the Nyloc nut prevents it from ever vibrating loose.

- Push the completed hub/arm assembly onto the motor's 3 mm D-shaft and tighten the set screw (or check the tight friction fit).

### Step 3: Rigid Sensor Mounting (ADXL345)

- Place the ADXL345 breakout board on the wooden base 15 mm to 25 mm away from the motor bearing face.

- Fasten the sensor board firmly to the wood using two small screws or a rigid 3D-printed clip.

> [!CAUTION]

> Prohibited: Do NOT use squishy double-sided foam tape or hot glue. Soft materials act as vibration dampers/shock absorbers that attenuate high-frequency signals and distort FFT readings.

### Step 4: Soldering the 1N4007 Flyback Diode (G3-3)

- Inspect the 1N4007 diode: Notice the body has a printed silver/white band at one end. That end is the Cathode.

- Solder the diode directly across the two electrical solder lugs on the back of the N20 motor:

* Cathode (Silver Band end): Solder to the Positive (+12V) motor terminal.

* Anode (Plain black end): Solder to the Negative (GND) motor terminal.

- Engineering Purpose: An electric motor is a large inductive coil. When the KCD1 switch is turned OFF, the collapsing magnetic field creates a reverse voltage spike (back-EMF) that can exceed 100V. The 1N4007 instantly clamps this spike safely, protecting your switch contacts and nearby sensor logic.

### Step 5: Wiring the DC Power Input Jack & Isolation

- Take the screw-terminal DC female jack:

* Connect the + terminal to one pin of the KCD1 Rocker Switch.

* From the other pin of the switch, wire to the inline fuse holder (carrying the 1A fuse).

* From the fuse holder, connect to the motor Positive (+12V) terminal.

* Connect the - terminal of the DC jack directly to the motor Negative (GND) terminal.

- Absolute Galvanic Isolation Rule:

* The 12V motor ground must NEVER touch the ESP32 ground pin.

* The ESP32 is powered exclusively via the 5V USB cable from the laptop.

* Testing rule: Touch multimeter probes between motor GND and ESP32 GND. The meter must read Open Circuit / Infinite Ohms ($\infty\ \Omega$).

### Step 6: Power Decoupling on the Breadboard (G3-1, G3-2, G3-4)

- Seat the 7Semi ESP32-DEVKIT-E firmly into the breadboard.

- Install the 100 µF Electrolytic Bulk Capacitor (G3-1):

* Identify polarity: The side with a white/grey stripe and minus signs --- is the Negative lead (shorter pin). The other lead is Positive.

* Insert the Positive lead into the breadboard row connected to the ESP32 3V3 pin (Pin 1).

* Insert the Negative lead into the breadboard GND rail (Pin 14).

* Keep leads trimmed short ($\le 5\text{ mm}$).

- Install the 100 nF Ceramic Capacitor (G3-2):

* Insert directly in parallel with the 100 µF capacitor across the 3V3 and GND rows (ceramic caps are non-polarized).

* Engineering Purpose: When the ESP32 initiates Wi-Fi calibration or high-speed processing bursts, current spikes to 500 mA. The 100 µF capacitor dumps stored charge in microseconds, preventing the AMS1117 LDO from dropping below the 2.70V Brownout threshold (rst:0x10).

- Install the 220 Ω Resistor (G3-4):

Insert in series with the Red anode pin of the RGB status LED connected to GPIO27. (Robu already supplied 330 Ω for Green/Blue).*

### Step 7: Transparent Safety Guard (G4-7)

- Take a small, sturdy transparent container (clear acrylic box or thick clear plastic cup).

- Place it over the spinning eccentric hub and motor shaft.

- Fasten it to the wooden base with 2 small screws or brackets.

- The 360° Clearance Test: With power disconnected, reach in and turn the motor shaft by hand one full circle ($360^\circ$). Ensure there is at least $5\text{ mm}$ of clear space between the rotating bolt/washers and the inside walls of the guard.

## 5. Pre-Power Safety Sign-Off (Amith's Checklist)

Before plugging in the 12V wall adapter or the USB cable:

☐ Locking Nut Checked: M3 Nyloc nut is threaded past the nylon collar on the eccentric bolt.

☐ Clearance Checked: Manually rotated $360^\circ$ — zero contact with baseplate or safety guard.

☐ Flyback Diode Checked: 1N4007 silver band is connected to the +12V motor terminal.

☐ Galvanic Isolation Checked: Multimeter confirms $\infty\ \Omega$ between 12V motor DC ground and ESP32 USB logic ground.

☐ Bulk Cap Polarity Checked: Negative stripe of 100 µF capacitor is connected to GND, not 3V3.

☐ Fuse Verified: 1A fast/time-delay cartridge fuse is inside the holder.